New York State Department of Health

Bureau of Community Environmental Health and Food Protection

Acceptable Encapsulant Products

# February, 2008

| Product Name | Manufacturer | Not Recommended | Dry Film Thickness | Respiratory Protection Required | Occupancy During an Application | States Approved In |
| --- | --- | --- | --- | --- | --- | --- |
| Back to Nature "Protect-A-Coat" | Dynacraft Industries Inc. Or Back to Nature Products Company 28 Harrison Avenue Englishtown, NJ 07726 (800) 922-0621 www.backtonatureprod.com | - Walls or ceilings with chalking or calcimine paint - Door jambs - Window headers and stops - Working radiators - Friction surfaces - Exterior surfaces | 7 mils minimum | For spray application, use molded polymesh shell dust and mist respirator - MSHA, NIOSH Approval No. TC-23C-287 or equivalent | - Occupants must be out during the day for spray application only. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application | CO, OH, MA, MI |
| Barrier Coat II | W.R. Grace & Co. 62 Whittemore Avenue Cambridge, MA 02140 (908) 531-3666 (800) 334-8796 www.na.graceconstruction.com | - Walls with chalking or calcimine - Door jambs - Window headers and stops - Ceilings with calcimine - Working radiators - Friction surfaces | 7 mils minimum (interior) 14 mils (exterior) | For spray application, use molded polymesh shell dust and mist respirator - MSHA, NIOSH Approval No. TC-21C-287 or equivalent | - Occupants must be out during the day for spray application. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application | CO, OH, MI |
| Insl-Cap Lead Encapsulating Compound | Insl-X Products Corp. 50 Holt Drive Stony Point, New York 10980 (845) 786-5000 www.insl-x.com | - Walls or ceilings with chalking or calcimine paint - Door jambs - Window headers and stops - Working radiators - Friction surfaces - Exterior surfaces | 7 mils minimum | For spray application, use molded polymesh shell dust and mist respirator - MSHA, NIOSH Approval No. TC-23C-287 or equivalent | - Occupants must be out during the day for spray application only. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application. | CO, OH, MA, MI |
| L-B-C Lead Barrier Compound (Type 1-Interior) | Fiberlock Technologies, inc 150 Dascomb Road Andover, Mass 01810 (800)342-3755 (978)623-9987 www.fiberlock.com | - Walls with chalking or calcimine that have not been prepared with a surface conditioning product - Door jambs - Window headers/stops - Ceilings with calcimine - Working radiators where Surface temperatures Intermittently exceed 240 degrees F. - Friction surfaces | 7 mils minimum | For spray application Use molded polymesh Shell dust and mist Respirator - MSHA, NOISH Approval No. TC-21C-287 or equivalent | - Occupants must be out during the day for spray application only. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application. | CO CT OH, MA |
| L-B-C Lead Barrier Compound (Type II - Exterior) | Fiberlock Technologies, Inc 150 Dascomb Road Andover, MA 01810 (800)342-3755 (978)623-9987 www.fiberlock.com | - Walls with chalking or calcimine that have not been prepared with a surface conditioning product - Door jambs - Window headers/stops - Ceilings w/calcimine - Working radiators where surface temperatures intermittently exceed 240 degrees F. - Friction surfaces | 7 mils minimum | For spray application, use molded polymesh shell dust and mist respirator - MSHA, NIOSH Approval No. TC-21C-287 or equivalent | - Occupants must be out during the day for spray applications only. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hrs. after appl. | CO, CT |
| L-B-C Lead Barrier Compound Type III (Interior & Exterior) Or Childguard (Interior & Exterior) | Fiberlock Technologies, Inc 150 Dascomb Road Andover, MA 01810 (800)342-3755 (978)623-9987 www.fiberlock.com | - Walls or ceilings with chalking or calcimine paint - Door jambs - Window headers and stops - Working radiators - Friction surfaces | 7 mils minimum | For spray application, use molded polymesh shell dust and mist respirator-MSHA, NIOSH Approval No. TC-23C-287 or equivalent | - Occupants must be out during the day for spray application only. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application. | CO, CT, OH, MI CT, OH, MA, MI |
| Lead-Master | Fiberlock Technologies, Inc 150 Dascomb Road Andover, MA 01810 (800)342-3755 (978)623-9987 www.fiberlock.com | - Walls with chalking or calcimine that have not been properly prepared with a surface conditioning product - Door jambs - Window headers and stops - Ceiling with calcimine - Working radiators where the surface temperatures intermittently exceed 240 degrees F. - Friction surfaces | 15 mils minimum | For spray application Use molded polymesh Shell dust and mist Respirator—MSHA, NOISH Approval No. TC-21C-287 or equivalent | - Occupants must be out during the day for spray application only. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application. | CO, CT, MA, MI |
| Lead Block | Coronado Paint Company, A division of Insul-X Products Corp. 2250 Arthur Avenue Elk Grove Village, IL 6007 (708) 439-4200 (800) 323-0633 www.coronadopaint.com | - Walls with chalking or calcimine - Door jambs - Window headers, stops, and sills - Ceilings with calcimine - Working radiators if not top coated with alkyd or latex enamel - Handrails and railing caps - Cabinets and drawers - Friction surfaces or removable closures | 14 to 16 mils | For spray application, use molded polymesh shell dust and mist respirator - MSHA, NIOSH Approval No. TC-23C-287 or equivalent | - Occupants must be out during the day for spray application. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application | CO, CT, OH, MI |
| LeadLock GE-40 | GLOBAL Encasement, Inc. PO Box 7724 North Bergen, NJ 07087 (201) 902-9770 (800) 266-3982 www.encasement.com | - Walls with chalking or calcimine - Door jambs - Window headers, stops and sills - Ceilings with calcimine - Working radiators if not top coated with alkyd or latex enamel - Handrails and railing caps - Cabinets and drawers - Friction surfaces or removable closures | 9 mils minimum for interior 18 mils minimum for exterior | For spray application Use molded polymesh Shell dust and mist Respirator—MSHA, NOISH Approval No. TC-21C-287 or equivalent | - Occupants must be out during the day for spray application. - Occupants may be in residence but not in room or work area undergoing encapsulation if - Use adequate ventilation during the first 24 hours after application | CO, CT, OH, MI |
| Lead Stop | Dumond Chemicals 1501 Broadway, 26th Floor New York, New York 10036 (212) 869-6350 Fax (212) 764-5762 www.dumondchemicals.com or www.peelaway.com | - High or constant friction areas such as window sash liners or door frames - Walls with chalking or calcimine - Door jambs - Window headers and stops - Ceilings with calcimine - Working Radiators - Frictions surfaces such as cabinets and drawers | 6 mils | Abide by OSHA regulations when using this product | - Occupants must be out during the day for spray application. - Occupants may be in residence but not in room or work area undergoing encapsulation if brush or roller is used. - Use adequate ventilation during the first 24 hours after application | CO, OH, MA, MI |

P:Sections\Residentual\LEAD\ENCAPSUL\ENCAPCH42108